CV512 TV program factory menu new system configuration criteria is part of the operating instructions (draft)

table of Contents

About entering the factory menu:

Press the Source button menu in the Source Input Press enter key 2580 shown in the main page factory menu Factory Setting, Factory Setting the factory menu page as shown below:

- Version Item :

The detailed information of the software, such as: Screen parameters, motherboard, compile time and other information, mainly used to query product information.

  - BootROM Ver  (boot version)

  - AudioRoM Ver( Audio Configuration version)

  - Panel name(Open Cell name)

  - Customer HW Ver（board model）

  - Customer SW Ver (SW numver)

  - FLASH Type(FLASH type)

  - OTA Ver (OTA version number)

  - CI KEY Host(CI key ID)

  - CI Key SN (CI key ID)

  - HDCP Key( HDMI HotPlus key version)

  - Checksum (production time)

  - VIP _Table Ver()

  - Builder (SW factory division name)

  - Build Time （SW version time）

Detailed display of various information software for product information query.

2. SystemConfig items：

SysteamConfig item is CV512L program factory main function menu item is used to configure the software system functions, customers can in this general configuration is complete, enter the menu display as follows:

  - BoardDefined：（fixed non-switchable）

  - DefaultLanguage：

Operation mode: the remote control of the Left [] / [] Right turning cycle selection button;

The selected OSD menu language is the default language of the software.

  - DefaultCountry：

Operation mode: the remote control of the Left [] / [] Right turning cycle selection button;

The selected country is the default country for the software.

  - OSD Language：

Operation: Press the remote control [Left] / [Right] key to enter the OSD language support detailed configuration options;

The function is used to configure the software to support the type of OSD language display options into the setting interface as follows:

Select off the language, you can not be selected in the main menu.

- Reset Default：

Operation mode: the remote control [of] Left / Right [key];

Reset function for the OSD configuration support, default after reset language: select only English, other languages are off by default.

- English：

Operation method: Left of the remote control [] / [] Right key;

OFF is shown as closed English, ON is selected for the English language OSD support of one.

Select the way other languages with English.

- Next Page:

Operation method: The remote control [OK] / [ENTER] / [] Right key;

This item does not display the language for the page function.

When press MENU returns the complete configuration language may return to               SysteamConfig function key.

  - DVD_Android_Option：

Operation mode: the remote control [of] Left / Right [key]

The use of an external functional module board configurations or DVD Android module configuration interface as follows:

- DVDInputPort：

Operation mode: the remote control of the Left [] / [] Right key to select cycle;

The DVD signal source for selecting the input channel corresponding to the Input sourc menu which channel Lable

For example, when a DVD YPBPR signals selected input channel, the program, all the YPBPR Lable corresponding display a DVD

- AndroidInputPort：

Operation mode: the remote control of the Left [] / [] Right button to cycle;

The signal source for selecting the input channel corresponding to Android Input sourc menu which channel Lable

When selected as the input channel signal for Android HMDI3 the program corresponding to all display Lable HMDI3 press MENU key to return to the main menu to set completion SysteamCongfig forAndroid.

  - 6M20_6M30_Option：

Operation mode: the remote control of [] Left / Right [key];

The adapter plate of the type used for LVDS signals transmitted to the panel board configurations, after entering the CI interface displays the following:

- TransformSupport：

Operation mode: the remote control of [] Left / Right [key];

The LVDS signal for selecting whether to support the adapter plate, OFF is not supported, ON support

- 6m20_6M30 Option：

Operation mode: the remote control of the Left [] / [] Right button to cycle;

The adapter plate for setting the LVDS signal type 6M20 6M30 or adapter plate.。

  - PreChannelSetting：

Operation mode: the remote control of the Left [] / [] Right button selection;

The frequency with only the production line is configured to preset a TV signal point information, are shown ATV, DVB-T and DVB-C three preset channel selection option is to enter, display interface as follows:

- ATVChannelSetting：

Operation mode: to select the remote control of the [MENU] [] Left / Right [BACK] key menu button ;

The analog signal is ATV channel parameter settings, select the specific set into the interface as follows:

i)                     Channel_Freq:

Operation mode: press number keys set frequency value, when the end of input parameters, the need to move the cursor to the remote control SaveCurrentChannel the [OK] / [ENTER] / [] Right key to save the preset frequency channels, or not to save Frequency information! Repeat the same procedure to add more preset channel, for setting the predetermined frequency of the signal frequency parameters, in Hz.

ii)                    AudioStand:

Operation mode: the remote control Left [] / [] Right key to select cycle;

The configuration preset channel sound formats, such as such as BG or DK.

iii)                  VideoStand

Operation mode: the remote control Left [] / [] Right key to select cycle;

The image format of default channel configuration, such as selecting PAL-I or SECAM system.

iv)                  SaveCurrentChannel:

Operation mode: the remote control of the [OK] / [ENTER] / [] Right key to select;

This is the frequency at which the preset channel is currently set.

v)                    ViewPreChannel:

Operation mode: the remote control of the [OK] / [ENTER] / [] Right key to select ;

The item is watch An already preset and save the preset channel .

vi)                  ViewNextChannel:

Operation mode: the remote control of the [OK] / [ENTER] / [] Right key to select;

This is the default channel that has been preset and saved.

vii)                AddNewChannel:

Operation mode: the remote control of the [OK] / [ENTER] is selected / Right [key];

The item is to add a new preset channel.

- DVBTChannelSetting：

DVB-T channel preset setting function is not implemented, indicating that temporarily missing!

- DVBCChannelSetting：

DVB-C channel setting preset function is not implemented, indicating that temporarily missing!

  - Power Show Logo：

Operation mode: the remote control Left [] / [] Right key to select;

The feature set whether to display the system boot LOGO, select Hide Logo Selecting the boot display LOGO SHOW LOGO; boot is not displayed LOGO.

  - AC Auto PowerOn：

Operation mode: the remote control Left [] / [] Right key to select;

To select ON, the electric power system;

Select OFF, the standby power system;

Select LASTMemory, memory boot, that is, when before power is turned on, will automatically turn on again; if the power is off before re-power is turned off.

  - Input Source：

Operation mode: the remote control Left [] / [] Right key to select, [MENU] Key to return to the previous menu;

This function is mainly used for breeding the version required to open the SOURCE port, enter the port configuration interface as follows:

- Reset Default：

Operation mode: the remote control of the [OK] / [ENTER] / [] Right key to select;

After the implementation of that reset function, the version with the port is automatically set as the default version number and type of the current port.

- ATV：

Operation mode: the remote control [of] Left / Right [Select], press [MENU] button to return to the previous menu;

When SOURCE ATV channel closes the channel when the select OFF, select ON, SOURCE ATV channel channel opening.。

- DTV、AT等等通道：

Other channels or play opening is closed with the ATV operation method;

- Next Page：

Operation mode: the remote control [OK] / [ENTER] / [] Right key to select;

This item is a page function item that shows other channel configuration items that are not currently displayed

  - PhysicalIputSetting：

Operation mode: the remote control [] Left / Right [enter] Press [MENU] button to return to the previous menu ;

This item is used to configure the physical signal input port of the channel to facilitate the replacement of the port signal input order. The detailed configuration interface is as follows:

a)        ResetDefault:

Operation mode: the remote control of the [OK] / [ENTER] / [] Right button to execute the reset function, press the [MENU] Key to return to the previous menu;

When the function is executed, the physical signal input order of the channel is the order of the default configuration of the company.

b)       YPBPR:

Operation method: The remote control of the [OK] / [ENTER] / [] Left / Right [] key to cycle;

Setting a UI YPBPR input source mapped to the input port of the motherboard, such as when the setting of the physical input ports to the board when PhyInputYbpr2, The signal input of YPBPR SOURC passage YPBPR2 input port to the board.

c)        Other HMDI, SCART, AV input port of the physical operation mode is provided with YBPBR.

  - Use CI 12 STACK Only：This lacking。

3.Picture Mode items：

Factory Setting the page surface, to move the cursor on the Picture Mode, press [OK] / [ENTER] / right enter Picture Mode page, shown below, press [EXIT] to exit the factory menu, press [MENU] button to return to the previous menu:

ADC Adjust：

The item is targeted Adjust the front under YPbPr, SCART, PC mode The ADC; When the three-way R / G / B or Y / Pb / Pr analog signal input to the main chip, due to variations in hardware, resulting into our signal range and the standard value of the chip is somewhat different, it is necessary The input signal is carried out ADC calibration, calibration image 100% colorbar required to ensure that the analog input signal into a main chip of our standard; Altogether R / G / B GAIN (gain amount) and R / G / B OFFSET (offset) six parameters.

a)        Mode:

Operation: According to the remote control OK / ENTER / left and right keys to select cycle;

The item is ADC Adjust channel selection function in PC-RGB channels, move the cursor to Adjust office,

Press the remote control [OK] / [ENTER] / right to automatic calibration, when Auto ADC Hints DONE When automatic calibration is successful; if tips Fail was automatically correct the failure.

b)       Adjust:

Operation mode: the remote control of the [OK] / [ENTER] / [] Left / Right [] key operation;

This item selects the function item for automatic calibration.

c)        R / G / B Gain:

Operation mode: the remote control of the [ Left] / [Right ] button adjustment value;

d)       R / G / B Offset:

Operation mode: the remote control of the Left [] / [] Right buttons adjust values;

e)        Other channels such as:

SCART, YPBPR Channel under the correct operation of the same way PC-RGA aisle.

W/B Adjust：

In Picture Mode page, the cursor will move to the W / B Adjust, press [OK] / [ENTER] / [Right] to enter the dark and white balance balance adjustment menu, by adjusting the R / G B GAIN (gain amount) / values and R / G / B oFFSET (offset) of the screen under the effect of improving the current channel. As shown below:

a)        Temperature:

Operation mode: the remote control of the Left [] / [] Right turning cycle selection button;

This item is for the color temperature mode option Standard / Cool / Warm mode three color temperatures.

b)       R / G / B GAIN:

Operation mode: the remote control of [] Left / Right [ ] button adjustment value;

c)        R / G / B OFFSET:

Operation mode: the remote control of the Left [] / [] Right buttons adjust values;

d)       The lack of Gamma Table

e)        Copy All: The temporary absence.

  - Picture：

In Picture Mode page, move the cursor to the Picture, press the [OK] / [ENTER] / [Right] Picture enter menu, as shown below:

a)        Mode:

Operation mode: the remote control of the Left [] / [] Right key to select turning cycle;

The item is Channels in Picture Mode selection function.

b)       PICTURE:

Operation mode: the remote control of the Left [] / [] Right key to select turning cycle;

The item is PICTURE Mode: dynamic (dynamic mode), Standard (standard mode), for the Mild (Soft mode), user (user mode).

c)        Brightness (luminance) / constrast (contrast) / color (saturation) / sharpness (sharpness) / tint (color):

Operation mode: the remote control of the Left [] / [] Right keys to adjust the value;

Options range above and the corresponding numerical values in each mode, can be adjusted according to manufacturer's standard test, in general, the lower the brightness (standard mode) Standard, contrast, saturation, sharpness are "50"; mild (soft And mode) of the four values of less than 50; four values under dynamic (dynamic mode) is generally greater than 50, so that the screen looks more beautiful; User (user-defined mode) values can be freely set in accordance with customer demand.

d)       Noise Reduction:

Operation: According to the remote control [] Left / Right [Select] button;

The items selected for noise reduction mode, there are four modes are available: Middle / High / Off / Low , generally set Middle mode.

e)        Color Matrix: Lack of function

f)         Dithering Level: Lack of function

g)        Dithering Algorithm 2D, 3D cross-cut.

  - OverScan：

In Picture Mode page, move the cursor to the Over Scan, press OK on the remote control [] / [ENTER] key to enter the menu, as shown below:

- Scanle Mode：

Operation mode: the remote control [of] Left / Right {} key selection;

The zoom ratio for the current Source function under the screen, generally have the following modes: 4: 3, 16: 9 , Auto, Panorama, Cinema, Zoom1, Zoom2, Just Scan and so on.

VTop-OverScan (scaling trim the vertical direction),

VBottom-OverScan (scaling trim in the vertical direction),

HLeft-OverScan (left to fine-tune the horizontal scaling),

HRight-OverScan (the right to fine-tune the horizontal scaling),

Ext Source H Start (coarse horizontal direction)

Ext Source V Start (coarse vertical direction)

Operation mode: the remote control [of] Left / Right adjustment {} key;

This item is the option for the recall rate parameter.

。

PS：

512 and 9203 ATV, AV, SCART-CVBS , NTSC under SCART-RGB signals, PAL, SECAM standard OVERSCAN three are independent;

YUV and HDMI are sub-resolution, with the best engineers to confirm the good, the value of re-commissioning, adjusted value can be sent to a software engineer.。

  - Curve Setting：

In Picture Mode page, move the cursor to the Curve Setting, press OK on the remote control [] / [ENTER] key to enter the menu, as shown below:

This is the curve setting: mainly set the nonlinear parameters and the size of the range.

a)          Source:

Operation mode: the remote control of the Left [] / [] Right key to select turning cycle;

This item corresponds to the curve setting Source Mode.

b)         Curve Type:

Operation mode: the remote control of the Left [] / [] Right key to cycle to the Source corresponding to the selected contrast Mode

(Contrast), Brghtness (brightness), Sharpness (clarity), Color (saturation) to choose.

c)         Curve Point 0:

Operation mode: the remote control [of] Left / Right [] are set numeric keys.

d)         Curve Point 50:

Operation mode: the remote control [of] Left / Right set values {} key.

e)         Curve Point 50:

This is the default value for the OSD, and the debugger typically debugs the value of this item

Operation mode: the remote control [of] Left / Right set values {} key.

f)          Curve Point75:

Operation mode: the remote control [of] Left / Right set values {} key.

g)         Curve Point 100:

Operation mode: the remote control [of] Left / Right { set} key values.

Gmma Switch  to be perfect；

Select PQ  ：function to be perfect；

  - Dynamic Contrast(动态对比度)

In Picture Mode page, move the cursor to the Dynamic Contrast, press [OK] / [ENTER ] key to enter the menu, as shown below:

Operation mode: the remote control [of] Left / Right {} key selection;

Dyn Contrast Level OFF points, the higher the low, middle, high fourth gear, the grade, the stronger the effect.

The rest of the value of the corresponding grade is the value of fine-tuning;

- PreColorFineTune ：

Operation mode: the remote control of the Left [] / [] to enter the Right, [MENU] button press RET:

The PQ is a common front end interface debug registers, as shown below:：

i)                     Mode:

Operation mode: the remote control of the Left [] / [] Right tumble bond selected, press the [MENU] to return to the previous menu;

The options for the Source Mode.

ii)                    Reset To Default:

Operation mode: the remote control of the Left [] / [] Right key to perform, press the [MENU] to return to the previous menu;

When this is performed, ACE_Contrast, ACE_Brightness, ACE_Saturation display the current channel pre-register value of the program will restore the default settings.

iii)                  ACE_Contrast:

Operation mode: the remote control [of] Left / Right [] key to cycle parameter, press the [MENU] Key to return to a superior menu;

The pre-PQ debug Contrast (contrast) register which is valid only for the analog channel, digital channel is invalid.

iv)                  ACE_Brightness:

Operation: press the remote control [] Left / Right [] key to cycle parameter, press the [MENU] Key to return to the previous menu list;

The pre-PQ debug the Brightness (brightness) register which is valid only for the analog channel, digital channel is invalid.

v)                    ACE_Saturaton:

Operation:: [Left] / [ Right ] key to cycle parameter, press the [MENU] Key to return to the previous menu;

The pre-Saturation, debug PQ (saturation) the register, the register is valid only for the analog channel, digital channel is invalid.

- PEAKING：

The professional registers and debugging methods PreColorFineTune consistent.

- CTI：

- Sound Mode item：

Factory Setting the page surface, to move the cursor on the Sound Mode, press The remote control [OK] / [ENTER] / Right] [ enter Sound Mode page, press the [MENU] button to return to the previous menu, as shown below:

- ACE_ColorFineTue:

Instructions: Left by the remote control [] / [] Right key to select cycle Standard (standard mode), Moive (cinema mode), Music (music mode), Speech (speech mode), User (with User mode), five sets of sound mode.

- Bass:

Instructions: [Left by remote] / [] Right key value adjustment;

BASS bass item is provided;

- Treble:

Instructions: [Left by remote] / [] Right key value adjustment;

Treble is set treble effect items.

- Volume:

Instructions for operation: press the remote control [OK] / [ENTER] key to access the submenu shown below, press [MENU] button to return to the previous menu;；

- OSD Volume Value：

Operation method: The remote control of the [OK] / [ENTER] value adjustment ;

The default value for the OSD volume.

- Volume Point0、Volume Point10 ~Volume Point100：

Operation mode: the remote control of the [OK] / [ENTER] value adjustment;

Option 11 or more sound curve is divided into 10 sections to be adjusted, Point Point 10 is less than the value 0, and so on.。

- Burning Mode item：

Operation: In the factory the main page menu, move the cursor to press Burning Mode [OK] / [ENTER] / [Right] key to perform;

The aging to enter the factory mode, the screen: red, green, blue, black, white color on the screen fifth alternating cycle presented by POWER key key pad may exit the aging mode.

- Shipping Mode item：

Operation: In the factory the main page menu, move the cursor to Burning Mode Press [OK] / [ENTER] / [] Right key to perform ;

The factory settings: Products factory, the factory will be used to this, empty except for some settings of the ADC.。

- Reset All items：

Operation mode: On the main menu page of the factory menu, move the cursor to Burning Mode press the [OK] / [ENTER] / [] Right key to perform;

The total reset for the factory, set all empty, after the implementation of automatic restart

- Function keys：

Factory Setting the main page, move the cursor to Function, according to the remote control [OK] / [ENTER] / [] Right key to go to the next page Function, press the [MENU] button to return to the previous menu, as shown below:

  - PanlSetting：

Operation mode: the remote control of [] Left / [Right] key to enter, press the [MENU] button to return to the previous menu;

This is the basic parameters for the screen when the screen configuration, as shown below:

- BackLight：

Operation: press the remote control Left [] / [] Right key loop regulation;

The backlight brightness value is debugging, the debug cycle is the debugging range values between 1-100.

- PWM_FREQ：

Operation mode: the remote control of the Left [] / [] Right key loop adjustment section

The motherboard PWM output frequency setting parameter, can be set within the range between 0-1023HZ.

- Pixel Clock：

Operation mode: the remote control [of] Left / Right adjustment {} key ;

The pixel clock screen parameters, the unit is MHz.

- Color Depth：

Operation method: Left of the remote control [] / [] Right key for loop regulation;

The color depth of the screen reference position, there are the following three modes: 6 bit, 8 bit, 10 bit.

- LVDS Channel Swap：

Left of the remote control [] / [] Right key to select the parity channel LVDS signal exchange function

- LVDS Converter：

Operation mode: the remote control [of] Left / Right circulation adjustment {} key;

The convert LVDS switching function, On open, Off Off.。

- LCDS Single Dual：

The switching function for the screen with LVDS single or double.

  - Hotel Mode：

Operation mode: the remote control [of] Left / cyclic selection ON / OFF] [Right key ;

The mode switching function for the hotel, ON hotel open mode, OFF off hotel mode.

  - SSC Adjust：

Operation mode: the remote control [of] Left / Right [ cyclic] key and adjust the value selection;

This item sets the function item for the screen.。

  - Top :

Adjust the tuner parameter value.

  - The RFAGC :

Adjust the value of the tuner.

  - Channel PRESET :

Operation mode: the remote control of the Left [] / [] Right key to select the ON / OFF operation;

The preset clearance card entry passage, when set to ON, the preset channel is opened, if set to OFF , the preset channel is closed.

  - Etected_RFFILTER D :

Move the cursor to Detected_RFFILTER, press OK on the remote control [] / [ENTER] / [Right] to enter the menu setting shown below:

  - C I Plus Key Import：

Operation: According to the remote control [OK] / [ENTER] / [] Right key is operated;

The CI plus for the import of KEY.

Host ID：

The ID CI of a number.

Serial Number：

The CI for the serial number.

- USB Upgrade item：

Operation mode: the remote control of the [OK] / [ENTER] / [Right] to enter the menu shown below:

The prerequisite for the USB upgrade: file to be upgraded in advance into the root of the U disk, U disk and into the USB port, the remote control The [OK] / [ENTER] / [Right] key to enter the menu shown on the map, select Yes to carry out the bin file for upgrade, select No, Exit all menus; currently CV512 program has done a USB auto-update feature, only will need to be upgraded bin file name is changed to rom.bin, is placed under the root directory of U, then U disk into the mouth USP, you can automatically upgrade.

Please do not power off during the upgrade, the upgrade process takes about 1 to 2 minutes, the system will automatically reboot after the upgrade.

6M20 Update item:

Operation: Operation mode: the remote control of the Left [] / [] Right key is operated;

For the 120HZ screen will feature a small plate (6M20 / 6M30), this is an option for 6M20 / 6M30 small board upgrade offer.

, UART DEBUG items:

Operation: According to the remote control [Left] / [Right] key to cycle turning to choose Debug / Aligmen / Auto modes.

, Import / Export_Data items

Operation: According to the remote control [Left] / [Right] key to perform the function, press the MENU key to return to the previous menu;

This function is mainly used for program data import and export functions, the current specific functions can be achieved for the following points:

1) Export SWImage: Software Function cloning;

2) Export Sysconfig: export system configuration data;

3) Import New Logo: Logo replacement function;

      - Export SWImage (Software clone features ):

Operation: press the remote control [Left] / [Right] key to perform the function, Press the [MENU] button returns to the previous level menu;

When performing this function, the system software will be cloned into the U disk file named file name CV_9202 generated for rom.bin, if a failure occurs clone proposed message "Fail" Success is "OK".

Note cloned merely mirror Image software itself, and does not include information data I door system configuration.

      - Sysconfig the Export (export system configuration data):

Operation method: Press the remote control Left [] / [] Right key executive function, press the [MENU] key to return to the previous menu ;

The data export function for the system, when performing this function, the system will clone software to the U disk file name generated for a file named CV_9202 CV_FACTORY_Data_Backup.bin , put forward message appears if cloning fails "Fail" Success is "OK" The

In order to ensure the accuracy of exported data, the data must upgrade guide out on the platform back test (next Import Sysconfig operation), if the upgrade is illustrated in the U disk production lead out of the data is correct, otherwise the data is wrong of.

      - Sysconfig Import (import system configuration data)

Operation method: Left of the remote control [] / [] Right key to perform the function, then press the [MENU] button return to the previous menu ;

Performing this function is to import data CV_FACTORY_Data_Backup.bin curing system configuration information into the FLASH.

      - Import New Logo (introducing new boot LOGO )

Operation method: Left of the remote control [] / [] Right key to perform the function, then press the [MENU] button return to the previous menu;

The new boot LOGO for the import function.